GRADE 10 ELECTRICAL TECHNOLOGY

PAST PAPERS DATABASE

Two full examination papers with complete marking schemes,

set in the standard Kenyan (KICD/CBC) examination format.

Contents:

Paper 1 — Mid-Term Examination (80 marks, 2 hours)

Paper 1 — Marking Scheme

Paper 2 — End of Term Examination (100 marks, 2½ hours)

Paper 2 — Marking Scheme

REPUBLIC OF KENYA

MINISTRY OF EDUCATION

GRADE 10

ELECTRICAL TECHNOLOGY

MID-TERM EXAMINATION

Fundamentals of Electricity, Electrical Machines & Electronics

TIME: 2 HOURS

TOTAL MARKS: 80

Name: …………………………………………………………

Admission No: ……………………………     Class: ……………………

School: …………………………………………………………

## Instructions to Candidates

- Write your name, admission number and class in the spaces provided above.

- This paper consists of TWO sections: Section A and Section B.

- Answer ALL questions in the spaces provided.

- All working must be clearly shown where applicable.

- Non-programmable calculators may be used.

- This paper consists of printed pages. Candidates should check to ascertain that all pages are printed as indicated and no questions are missing.

# SECTION A: SHORT ANSWER QUESTIONS (40 MARKS)

Answer ALL questions in this section.

1. State four uses of electricity in day-to-day life.  (4 mks)

2. Define the term 'capacitance' and state its SI unit.  (2 mks)

3. State Ohm's Law.  (2 mks)

4. Distinguish between a primary cell and a secondary cell.  (2 mks)

5. Name two items of Personal Protective Equipment (PPE) used by an electrician.  (2 mks)

6. Two resistors of 6Ω and 12Ω are connected in parallel. Calculate the total (equivalent) resistance.  (3 mks)

7. State the right-hand grip rule.  (2 mks)

8. Distinguish between an N-type and a P-type semiconductor.  (2 mks)

9. State two methods of demagnetising a magnetic material.  (2 mks)

10. Name the three terminals of a bipolar junction transistor.  (3 mks)

11. Distinguish between isometric and oblique pictorial drawing.  (2 mks)

12. State two factors that determine the strength of the magnetic field of a solenoid.  (2 mks)

13. Three capacitors of 2µF, 4µF and 6µF are connected in series. Calculate the total capacitance.  (3 mks)

14. State the function of a Residual Current Device (RCD) in a domestic wiring installation.  (2 mks)

15. State two applications of a Zener diode.  (2 mks)

16. Explain the meaning of the term 'doping' as applied to semiconductors.  (2 mks)

17. Define internal resistance of a cell.  (2 mks)

18. State two safety precautions to observe when charging a lead-acid battery.  (2 mks)

# SECTION B: STRUCTURED QUESTIONS (40 MARKS)

Answer ALL questions in this section.

19. The diagram below (not shown) represents a resistor-capacitor charging circuit connected to a 12V D.C supply.  (null mk)

(a) Sketch and describe the shape of the graph of voltage against time as the capacitor charges.  (4 mks)

(b) Define the term 'time constant' of an RC circuit.  (2 mks)

(c) State two factors that determine the time constant of an RC circuit.  (2 mks)

(d) State three applications of capacitors in electrical appliances.  (3 mks)

20. With reference to electromagnetism:  (null mk)

(a) Explain the principle of electromagnetism.  (3 mks)

(b) Describe the working principle of an electric bell.  (4 mks)

(c) State two faults that may occur in an electromagnetic relay and how each may be detected using a multimeter.  (4 mks)

21. Concerning semiconductor diodes:  (null mk)

(a) Draw and label the circuit symbol of a junction diode, showing the anode and cathode.  (2 mks)

(b) Distinguish between forward bias and reverse bias of a diode.  (4 mks)

(c) Describe a procedure for testing a diode using an analogue multimeter, and state the expected results for a good diode.  (4 mks)

(d) Distinguish between half-wave and full-wave rectification.  (4 mks)

GRADE 10 ELECTRICAL TECHNOLOGY

MID-TERM EXAMINATION — MARKING SCHEME

# SECTION A

1. Any four of: lighting; running domestic appliances; running industrial machines; communication; medical equipment; transport signalling; irrigation/agriculture. (1 mk each)

2. Capacitance – the ability of a capacitor to store charge. Unit: Farad (F). (1 mk each)

3. Current through a conductor is directly proportional to the potential difference across it, provided temperature/physical conditions remain constant. (2 mks)

4. Primary cell – cannot be recharged (1 mk); Secondary cell – can be recharged (1 mk).

5. Any two of: insulated gloves; safety boots; helmet; goggles; insulated tools; overalls. (1 mk each)

6. 1/RT = 1/6 + 1/12 = 2/12+1/12 = 3/12 → RT = 12/3 = 4Ω (3 mks: formula 1, substitution 1, answer 1)

7. If a current-carrying conductor is gripped with the right hand with the thumb pointing in the direction of current flow, the curled fingers indicate the direction of the magnetic field. (2 mks)

8. N-type – majority carriers are electrons, formed by doping with a pentavalent impurity (1 mk); P-type – majority carriers are holes, formed by doping with a trivalent impurity (1 mk).

9. Any two of: hammering while aligned E-W; heating above Curie temperature; placing in an AC solenoid and slowly withdrawing. (1 mk each)

10. Base, Collector, Emitter (1 mk each)

11. Isometric – axes at 120° to each other/30° to horizontal, true measurements along all axes (1 mk); Oblique – front face true shape, receding lines at an angle (usually 45°), often to reduced scale (1 mk).

12. Any two of: number of turns; amount of current; type of core material; length of coil. (1 mk each)

13. 1/CT = 1/2+1/4+1/6 = 6/12+3/12+2/12 = 11/12 → CT = 12/11 ≈ 1.09µF (3 mks: formula 1, substitution 1, answer 1)

14. Detects earth-leakage/imbalance between live and neutral current and rapidly disconnects the supply to protect against electric shock. (2 mks)

15. Any two of: voltage regulation/stabilisation; overvoltage protection; reference voltage source. (1 mk each)

16. Doping is the deliberate addition of controlled impurity atoms to a pure semiconductor to alter (increase) its electrical conductivity/create majority carriers. (2 mks)

17. The resistance within a cell itself that causes a voltage drop when current flows (opposes current flow inside the cell). (2 mks)

18. Any two of: charge in well-ventilated area; avoid short-circuiting terminals; keep away from naked flame/sparks; use correct charging rate. (1 mk each)

# SECTION B

Question 19

(a) Voltage rises exponentially from 0V, increasing rapidly at first then levelling off/approaching 12V asymptotically (curve description 2 mks; correctly shaped sketch 2 mks).

(b) Time constant – the time taken for the voltage/charge to rise to about 63.2% of its final (maximum) value. (2 mks)

(c) Resistance (R) and Capacitance (C) of the circuit — τ = RC. (1 mk each)

(d) Any three of: power factor correction; motor starting; smoothing/filtering in power supplies; tuning circuits; camera flash storage. (1 mk each)

Question 20

(a) Whenever current flows through a conductor a magnetic field is produced around it; this field can be used to do useful work (magnetise materials, produce force/motion). (3 mks)

(b) Current flows through the electromagnet coil, attracting the soft-iron armature which pulls the hammer to strike the gong; this movement breaks the contact at the make-and-break point, de-energising the coil; the spring pulls the armature back, remaking contact and repeating the cycle, producing continuous ringing. (4 mks – 1 mk per correctly sequenced stage)

(c) Open-circuit coil — detected by an infinite/very high resistance reading on the ohmmeter (no continuity) (2 mks); Short-circuited turns — detected by an abnormally low resistance reading compared to the rated value (2 mks). (Any other valid fault e.g. burnt contacts accepted)

Question 21

(a) Triangle pointing towards a bar; anode at the flat side of the triangle, cathode at the bar. (2 mks for correct symbol with correct labels)

(b) Forward bias – positive terminal to anode (P-side), negative to cathode (N-side); narrows depletion region; conducts once barrier potential is overcome (2 mks). Reverse bias – positive terminal to cathode, negative to anode; widens depletion region; blocks current except small leakage current (2 mks).

(c) Set multimeter to resistance/diode-test range; connect probes across the diode one way, then reverse; note readings (2 mks). A good diode shows low resistance/conduction in one direction (forward) and very high resistance/no conduction in the other (reverse) (2 mks).

(d) Half-wave rectification uses one diode and passes only one half-cycle of the AC waveform, giving a pulsating DC with gaps between pulses (2 mks). Full-wave (bridge) rectification uses four diodes and utilises both half-cycles, giving a smoother pulsating DC output with pulses closer together (2 mks).

REPUBLIC OF KENYA

MINISTRY OF EDUCATION

GRADE 10

ELECTRICAL TECHNOLOGY

END OF TERM EXAMINATION

Electrical Installation, D.C Circuits, Magnetism & Electronics

TIME: 2½ HOURS

TOTAL MARKS: 100

Name: …………………………………………………………

Admission No: ……………………………     Class: ……………………

School: …………………………………………………………

## Instructions to Candidates

- Write your name, admission number and class in the spaces provided above.

- This paper consists of TWO sections: Section A and Section B.

- Answer ALL questions in the spaces provided.

- All working must be clearly shown where applicable.

- Non-programmable calculators may be used.

- This paper consists of printed pages. Candidates should check to ascertain that all pages are printed as indicated and no questions are missing.

# SECTION A: SHORT ANSWER QUESTIONS (50 MARKS)

Answer ALL questions in this section.

1. State three methods used to generate electrical power in Kenya.  (3 mks)

2. Explain why electrical power is transmitted at high voltage over long distances.  (3 mks)

3. Name four pieces of equipment found at a consumer's intake point.  (4 mks)

4. Distinguish between a ring final circuit and a radial final circuit.  (4 mks)

5. State the function of a lightning arrestor in a substation.  (2 mks)

6. A circuit consists of two resistors of 5Ω and 15Ω connected in series to a 40V supply. Calculate: (a) the total resistance (b) the current flowing in the circuit.  (4 mks)

7. Distinguish between a step-up and a step-down transformer/substation.  (2 mks)

8. State two types of energy meters used by electricity distribution companies.  (2 mks)

9. Explain the domain theory of magnetism.  (3 mks)

10. State two ways of storing permanent magnets to prevent loss of magnetism.  (2 mks)

11. Define inductance and state its SI unit.  (2 mks)

12. Two inductors of 4H and 12H are connected in parallel. Calculate the total inductance.  (3 mks)

13. State two applications of an inductor in electrical circuits.  (2 mks)

14. Distinguish between self-induction and mutual induction.  (2 mks)

15. State the cut-off and saturation conditions of a transistor.  (2 mks)

16. Explain the term 'depletion region' in a P-N junction.  (2 mks)

17. State two safety precautions to be observed when working on a live electrical installation.  (2 mks)

18. A kettle rated 240V, 3kW is used for 20 minutes. Calculate the energy consumed in kWh.  (3 mks)

19. State two factors considered when selecting a cable size for a final circuit.  (2 mks)

20. State two functions of an isolator in a substation.  (2 mks)

# SECTION B: STRUCTURED QUESTIONS (50 MARKS)

Answer ALL questions in this section.

21. With reference to electrical distribution:  (null mk)

(a) Describe the path of electrical power from the generating station to a domestic consumer, naming the main stages.  (5 mks)

(b) State four pieces of equipment found in a substation and give the function of each.  (8 mks)

(c) Explain two advantages of underground cable distribution over overhead line distribution.  (4 mks)

22. Concerning final circuits and protection:  (null mk)

(a) Explain, with reasons, why a cooker circuit is usually wired as a dedicated radial circuit.  (4 mks)

(b) Distinguish between a fuse, a Miniature Circuit Breaker (MCB) and a Residual Current Device (RCD) as used for circuit protection.  (6 mks)

(c) State three factors considered when designing a final circuit for a building.  (3 mks)

23. Regarding magnetism and electromagnetism:  (null mk)

(a) State three factors that determine the strength of the magnetic field produced by a solenoid.  (3 mks)

(b) Explain, with a labelled sketch description, the magnetic field pattern around a straight current-carrying conductor.  (4 mks)

(c) Describe the working principle of a relay and state one application.  (4 mks)

(d) Give two reasons why an electromagnet is preferred to a permanent magnet in a scrap-metal lifting crane.  (2 mks)

GRADE 10 ELECTRICAL TECHNOLOGY

END OF TERM EXAMINATION — MARKING SCHEME

# SECTION A

1. Any three of: hydro-electric; geothermal; thermal; wind; solar; nuclear; biomass. (1 mk each)

2. High voltage reduces current for a given power (P=VI), which reduces I²R power losses in the lines and allows use of smaller/cheaper conductors. (3 mks: correct reasoning)

3. Any four of: service cable; sealing/cut-out chamber; energy meter; consumer unit/distribution board/main switch; earthing terminal. (1 mk each)

4. Ring circuit – socket outlets wired in a loop starting and ending at the distribution board, current shared by two paths, uses smaller cable (2 mks). Radial circuit – single cable runs from board through sockets in series, ending at the last socket, not looped back (2 mks).

5. Protects substation equipment from high-voltage surges caused by lightning strikes by diverting the surge safely to earth. (2 mks)

6. (a) RT = 5+15 = 20Ω (2 mks). (b) I = V/RT = 40/20 = 2A (2 mks).

7. Step-up – increases voltage (used at generating station for efficient transmission) (1 mk); step-down – decreases voltage (used before distribution to consumers) (1 mk).

8. Any two of: electromechanical/induction disc meters; digital/electronic meters; prepaid meters. (1 mk each)

9. A magnetic material consists of tiny regions (domains) each acting like a small magnet; in an unmagnetised state the domains point in random directions and cancel out; magnetising aligns the domains in the same direction, producing a net magnetic effect. (3 mks)

10. Any two of: use soft-iron keepers with unlike poles adjacent; store in N-S direction; avoid dropping/hammering/heating. (1 mk each)

11. Inductance – the property of a coil that opposes change in current flowing through it. Unit: Henry (H). (1 mk each)

12. 1/LT = 1/4 + 1/12 = 3/12+1/12 = 4/12 → LT = 12/4 = 3H (3 mks: formula 1, substitution 1, answer 1)

13. Any two of: chokes; transformers; filtering circuits; tuning circuits; relays/solenoid valves. (1 mk each)

14. Self-induction – EMF induced in a coil due to change of current in the same coil (1 mk); Mutual induction – EMF induced in a coil due to changing current in a neighbouring coil (1 mk).

15. Cut-off – both junctions reverse biased, transistor fully OFF, no collector current (1 mk); Saturation – both junctions forward biased, transistor fully ON, maximum collector current (1 mk).

16. The region at a P-N junction depleted of free charge carriers due to diffusion, creating a potential barrier that opposes further current flow. (2 mks)

17. Any two of: isolate/switch off supply before working; use insulated tools/PPE; use a voltage tester to confirm circuit is dead; work with dry hands/in dry conditions. (1 mk each)

18. Energy = P × t = 3kW × (20/60)h = 3 × 0.333 = 1kWh (3 mks: formula 1, substitution 1, answer 1)

19. Any two of: current-carrying capacity of the cable; length of run/voltage drop; type of insulation; ambient temperature/installation method. (1 mk each)

20. Any two of: allows disconnection of equipment for maintenance; isolates faulty sections; operated only under no-load/no-fault conditions (not for breaking load current). (1 mk each)

# SECTION B

Question 21

(a) Generation at power station (11-33kV) → step-up transformer at generating substation → transmission at high voltage (e.g. 132kV/220kV) via transmission lines → step-down at receiving/primary substation → sub-transmission/distribution at 33kV/11kV → step-down at distribution transformer to 415V/240V → service line to consumer's intake point. (5 mks – 1 mk per correctly ordered stage, max 5)

(b) Any four with correct function, 2 mks each: Transformers – step up/down voltage; Circuit breakers – protect against overload/fault currents by automatically disconnecting supply; Isolators – disconnect equipment for maintenance under no-load; Lightning arrestors – protect equipment from voltage surges; Busbars – common connection points for incoming/outgoing lines.

(c) Any two, 2 mks each: not affected by weather (wind, storms); more aesthetically acceptable/less visual obstruction; less risk of accidental contact/vandalism; more reliable supply (fewer weather-related faults).

Question 22

(a) A cooker draws a high current due to its high power rating; a dedicated radial circuit with appropriately sized cable and a cooker control unit safely handles this demand without overloading a shared ring circuit. (4 mks)

(b) Fuse – a wire/element that melts and breaks the circuit when current exceeds a safe value; must be replaced after operating (2 mks). MCB – an automatic switch that trips when current exceeds rated value; can be reset after fault clearance (2 mks). RCD – detects imbalance between live and neutral current (earth leakage) and rapidly disconnects supply to protect against electric shock, rather than against overcurrent (2 mks).

(c) Any three, 1 mk each: total connected load/diversity factor; cable current-carrying capacity and voltage drop; type/rating of protective device; length of cable run; nature and environment of installation.

Question 23

(a) Number of turns of coil; amount of current flowing; type of core material (and length of coil). (1 mk each, any three)

(b) The magnetic field forms concentric circles around the conductor, centred on the conductor; the direction is given by the right-hand grip/screw rule; field strength decreases with distance from the conductor and increases with current. (4 mks for correct, complete description)

(c) A relay uses a small control current through an electromagnet coil to attract an armature, which closes/opens a separate set of contacts, switching a separate (often higher-power) circuit (3 mks). Application: e.g. switching a motor/high-current circuit from a low-power control signal, automotive relays, etc. (1 mk)

(d) Any two, 1 mk each: can be switched on/off; field strength can be varied by changing current/turns; polarity can be reversed; allows controlled release of load.